Introduction

Customers and channels shall use this document when filling in their RFQ for a small volume prover (SVP)

Customer Name: ODS Metering Systems

Date:

Project Name:

Quote Number:

Example : inlet and outlet flanges facing right, controller at right side facing right (default)

Type of Flowmeters used (check which apply)

| type | check | Maker if known | Min flow rate | Max flow rate | Normal flow rate |
| --- | --- | --- | --- | --- | --- |
| Turbine |  |  |  |  |  |
| PD meter |  |  |  |  |  |
| Coriolis |  |  |  |  |  |
| Ultrasonic |  |  |  |  |  |

Flow rate unit:                                   BBL/H        USGPM        L/min        m3/h

Process and Design temperature (contact the sales office if lower than -40 °C (-40 °F) or higher than 80 °C (176 °F)

| Operating (Process) Temperature | Range: Min Max Normal | °F | °C |
| --- | --- | --- | --- |
| Design temperature | Range: Min Max | °F | °C |

Process and design Pressures

| Operating Pressure Range: Design Pressure Range: | Range: Min Max Normal Range: Min Max | psig | kPa | bar |
| --- | --- | --- | --- | --- |

Note: For Design temperatures above 37.8°C (100°F), pressure must be de-rated, following pressure standard ANSI B16.5

Process Viscosity Data: (Contact Sales Office for products above 100 cSt)

| Viscosity at max/min operating temperature: | Density [kg/m3]: | Flow rate |
| --- | --- | --- |

Calibration reference data:

Reference Temperature for Prover Calibration:   60 °F	  15 °C          20 °C 	    Other:

Units for Prover Calibration and transmitters:	      °C, bar	   °C, kPa       °F, psi

1.0 Model and maximum flow rate (choose one)

| SVP Model |  | Max flowrate (PD and Turbine meters) |  |  | Max flowrate (Coriolis & US meters) Note 1 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BBL/h | USGPM | m³/h | BBL/h | USGPM | m³/h |
| SVP005 |  | 715 | 500 | 114 | 390 | 273 | 62 |
| SVP015 |  | 2140 | 1498 | 340 | 1560 | 1092 | 248 |
| SVP025 |  | 3570 | 2499 | 568 | 1560 | 1092 | 248 |
| SVP035 |  | 5000 | 3500 | 795 | 1950 | 1365 | 310 |
| SVP050 |  | 7200 | 5040 | 1145 | 3113 | 2179 | 495 |
| SVP085 |  | 12500 | 8750 | 1987 | 5843 | 4090 | 929 |
| SVP120 |  | 17500 | 12249 | 2782 | 9353 | 6545 | 1487 |

Note 1: For coriolis and ultrasonic flow meters the flowmeter vendors recommend a minimum pass time of more than 1 sec. This effectively reduces the capacity of Small Volume Provers if one pass=one run is used, if multiple passes per run is used, the max flowrate remains as for turbine and PD meters.

| 1.1 Type of installation and material (choose one) | Process fluid wetted parts AISI 304/304L | Process fluid wetted parts AISI 316/316L |
| --- | --- | --- |
| Standard prover for onshore installation |  |  |
| Portable prover with trailer supplied |  |  |
| Offshore prover: 316 Stainless Steel for wetted parts and for drive end mechanical components (chains, bars, sprockets etc) |  |  |
| Portable with Offshore combination |  |  |

| 1.2 ASME/ANSI B16.5 Flange Rating (choose one) |  | 005 | 015 | 025 | 035 | 050 | 085 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1A | Class 150 RF connection flanges |  | Y | Y | Y | Y | Y | Y |
| 2B | Class 300 RF connection flanges |  | Y | Y | Y | Y | Y | Y |
| 3C | Class 600 RF connection flanges | Y |  | Y | Y | Y | Y | Y |
| 4D | Class 900 RF connection flanges | Y |  | Y | Y | Y | Y |  |
| 6A | Class 150 RJ connection flanges |  | Y | Y | Y | Y | Y | Y |
| 7B | Class 300 RJ connection flanges |  | Y | Y | Y | Y | Y | Y |
| 8C | Class 600 RJ connection flanges | Y |  | Y | Y | Y | Y | Y |
| 5D | Class 900 RJ connection flanges | Y |  | Y | Y | Y | Y |  |

1.3 Inlet/outlet orientation (choose one)

| 01 | Inlet and outlet flange opposite - inlet right side |  |  |
| --- | --- | --- | --- |
| 02 | Inlet and outlet flange same side - right side |  |  |
| 03 | Inlet and outlet flange 90°, inlet on right side and outlet on top |  |  |
| 04 | Inlet and outlet flange same side - left side |  |  |
| 06 | Inlet and outlet flange both on top |  |  |
| 07 | Inlet and outlet flange opposite - inlet left side |  |  |
| 09 | Inlet flange on top & outlet on left |  |  |
| 11 | Inlet flange on top and outlet on right |  |  |
| 12 | Inlet flange on left and outlet flange on top |  |  |

1.4 Safety ratings in Hazardous Areas and motor voltage (choose one)

| CSA | 005 | 015 | 025 | 035 | 050 | 085 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 VDC |  |  |  |  |  |  |  |
| 120 VAC 60 Hz | * | * | * | * | * |  |  |
| 220 VAC 60 Hz | * | * | * | * | * |  |  |
| 220/240 VAC, 60 Hz, 3 phase |  |  |  |  |  |  |  |
| 460/480 VAC, 60 Hz, 3 phase |  |  |  |  |  |  |  |

Standard : Available -20°C to +40°C (-4°F to +104°F) or -40°C to +40°C (-40°F to +104°F)  *Only available from -20°C to +40°C (-4°F to +104°F)

| ATEX or IECEx(Note-1) | 005 | 015 | 025 | 035 | 050 | 085 | 120 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 24 VDC | ** | ** | ** | ** |  |  |  |
| 220 VAC, 50 Hz |  |  |  |  |  |  |  |
| 220/240 VAC, 50 Hz, 3 Ph | *** | *** | *** | *** | *** | *** |  |
| 380/400/415 VAC, 50 Hz, 3 Ph |  |  |  |  |  |  |  |
| 460/480 VAC, 60 Hz, 3 Ph |  |  |  |  |  |  |  |
| 690 VAC, 60 Hz, 3 Ph |  |  |  |  |  |  |  |

Standard : Available -20 °C to +40°C (-4°F to +104°F) or -40°C to +40°C (-40°F to +104°F) or -20°C to +60°C    **Only ATEX, no IECEx available    ***Only available -20°C to +40°C (-4°F to +104°F) or -20°C to +60°C (-4°F to +140°F) ;

For other combinations of Voltages, Frequencies and Approvals, please contact the Honeywell sales office.

Note-1, sub assembly level certificates are provided
.

1.5 & 2.1  Safety ratings in Hazardous Areas and ambient temperature ranges (Choose one)

| CSA/US Class I, Div.1, Group D T2C* | -20 °C to +40 °C | -40 °C to +40 °C |  |
| --- | --- | --- | --- |
| CSA/US Class I, Div.1, Group C T3B* | -20 °C to +40 °C | -40 °C to +40 °C |  |
| ATEX II 2 (1) G Ex d [ia Ga] IIB T4 (T3) Gb, II 2 G c IIB T4 (T3) | -20 °C to +40 °C | -40°C to +40 °C | -20 °C to +60 °C |
| IECEx (Note-1) | -20 °C to +40°C | -40 °C to +40 °C | -20 °C to +60 °C |

*Not available on offshore provers

Note-1, available on the components level IECEx certificate.

1.6 Flow tube finish (choose one)

| Standard, Stainless steel brushed |  |
| --- | --- |
| White paint (single coat) |  |
| Other paint on request (please specify; YYY option table 6 to be completed) |  |

3.1 Water draw / Calibration (choose one)

| Standard, Gravimetric water draw according to API MPMS 4.9.4 |  |
| --- | --- |
| Volumetric water Draw according to API MPMS 4.9.2 |  |

3.2 Nitrogen purging of drive end for cold products, -40°C to -10°C (-40°F to +14°F) (choose one)

| Standard, Without Nitrogen purge system |  |
| --- | --- |
| Nitrogen purge system, Automatic (not available below ‐20 °C ambient temperature) |  |
| Nitrogen purge system, Manual |  |

3.3 Insulation (choose one)

| Standard, Without insulation |  |
| --- | --- |
| Insulation plate |  |
| Insulation jacket and plate |  |

4.1 NACE requirement (choose one)

| Standard, No NACE |  |
| --- | --- |
| NACE MR-0175 (ISO 15156) for H2S containing applications |  |

4.2 Material of seals (choose one)

| Standard Ekonol® filled PTFE seal |  |
| --- | --- |
| Carbon Fiber Reinforced PTFE seal (crude oil) |  |

4.3 Location and Power of Controller (choose one)

| AC voltage (With AC/DC converter) Right side facing right (check the drawing on top of this document) |  |
| --- | --- |
| AC voltage (With AC/DC converter) Left side facing left |  |
| 24 VDC power supply; Right side facing right (check the drawing on top of this document) |  |
| 24 VDC power supply; Left side facing left |  |

5.1 Pressure and Temperature transmitters (choose one)

| Standard, Honeywell Transmitters (Models; ST700 and STT750) |  |
| --- | --- |
| Rosemount Transmitters (Models; 3051T and 3144P) |  |
| Other transmitters (please specify, YYY option table 6 to be completed) |  |
| No transmitters |  |

5.2 PED Approval (choose one)

| Standard, No approval |  |
| --- | --- |
| PED (Pressure Equipment Directive, EU requirement) |  |

5.3 PMI (Positive Material Identification)  (choose one)

| Standard, No PMI |  |
| --- | --- |
| PMI (X-ray Fluorescence) on welds and pressurized parts |  |

6 Other Comments/requirements (incl. special YYY requirements here):

7 Crating (choose one)

| No Packing/ crating |  |
| --- | --- |
| Standard, Sea-worthy Wooden Crate |  |
| Blocking & Securing in Sea-going Container (when not possible to be crated, i.e., on trailer: container to be provided by client) |  |
| Other: (please specify, YYY option: table 6 to be completed) |  |

8 Other Options (choose all which apply)

| Battery pack/power supply unit (only possible for use with 24 VDC motor and controller) |  |
| --- | --- |
| Leak detection instrumentation consists of a precision differential pressure gauge and SS hoses |  |
| Water Draw Kit; Power supply, cables, and solenoid valve assembly. |  |
| Water Draw Instrumentation Kit: high accuracy thermometers and pressure gauge (NIST traceable). |  |
| Set of spare parts, 2 years (complete seals kit, set of optical switches, motor stop switch, motor relay). |  |
| Commissioning: Onsite start-up/commissioning by Honeywell Enraf service technician and/or authorized service provider |  |
| Saudi Aramco compliant version |  |
| Det Norske Veritas Offshore compliant version |  |
| Bureau Veritas Marine Offshore compliant version |  |

Other information

Product name or Type of Process Fluid: (Please supply relevant Material Safety Data Sheet)

Brand of Proving Computer used at site.

| OMNI | ABB/Spirit | Daniel/Emerson | Krohne | Dynamic | Virtual | Other |
| --- | --- | --- | --- | --- | --- | --- |

Name Plate and Tag name Information:

| Customer name to be displayed on Name Plates: | No / Yes; |  |
| --- | --- | --- |
| Process fluid to be displayed on Name Plates: | No / Yes; |  |
| Tag Number for Motor: |  |  |
| Tag Number for Pressure Transmitter: |  |  |
| Tag Number for Thermowell: |  |  |
| Tag Number for Temperature Transmitters: | Switch Bar Temp Transmitter: | Prover Temp Transmitter: |

Standard Prover Documentation

| Document | Document Schedule |
| --- | --- |
| Drawings (Outline, frame mounting, service clearance, wiring) | Two (2) weeks AAO * |
| FAT procedure | Two (2) weeks AAO * |
| Inspection and Test Plan | Two (2) weeks AAO * |
| Honeywell Enraf SVP Operation and Service Manual | Two (2) weeks after shipment |
| Hydrostatic Pressure Test Certificate | Two (2) weeks after shipment |
| Dielectric Strength Test Certificate | Two (2) weeks after shipment |
| Flow Tube Material Certificate | Two (2) weeks after shipment |
| Major Wetted Parts Material Certificates | Two (2) weeks after shipment |
| Hazardous Electrical Component Certificate | Two (2) weeks after shipment |
| Instrument Calibration Certificates (Includes weights, thermometers, PI-tape, pressure gauge, ID micro meter, transmitters and ground bond testing) | Two (2) weeks after shipment |
| Instrument Calibration Certificates for temperature and pressure instruments (if supplied with the SVP) | Two (2) weeks after shipment |
| Transmitter Manuals (if supplied with the SVP) | Two (2) weeks after shipment |
| Welder Qualifications | Two (2) weeks after shipment |
| Welding Procedures | Two (2) weeks AAO * |
| *AAO is After Acceptance of Order by Honeywell Engineering and Honeywell Order Management |  |

Distributor and/or Customer Information:

| Sales Contact: |  | Phone: |  | Fax: |  |  | Email: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Technical Contact: |  | Phone: |  | Fax: |  |  | Email: |  |
| Project Manager: |  | Phone: |  | Fax: |  |  | Email: |  |
| End User: |  |  | End User Location (Country): |  |  |  |  |  |

Distributor/

Customer Signature: ____________________________		Date: ________________________